| R1 ! ip vrf cia description CIA Intel rd 1:1 route-target export 1:1 route-target import 1:1 ! ip vrf fbi description FBI Intel rd 1:2 route-target export 1:2 route-target import 1:2 ! interface FastEthernet0/0 bandwidth 10000000 ip address 10.53.100.1 255.255.255.0 ip pim sparse-mode no ip mroute-cache duplex auto speed auto ! interface FastEthernet0/1 bandwidth 10000000 ip address 10.53.112.1 255.255.255.0 ip pim sparse-mode no ip mroute-cache duplex auto speed auto ! interface FastEthernet0/1.11 encapsulation dot1Q 11 ip vrf forwarding cia ip address 192.168.12.1 255.255.255.0 ! interface FastEthernet0/1.12 encapsulation dot1Q 12 ip vrf forwarding fbi ip address 172.30.12.1 255.255.255.0 ! router eigrp 1 network 0.0.0.0 no auto-summary ! router eigrp 2 auto-summary ! address-family ipv4 vrf cia network 192.168.0.0 0.0.255.255 no auto-summary autonomous-system 2 exit-address-family ! router ospf 1 vrf fbi log-adjacency-changes network 0.0.0.0 255.255.255.255 area 0 ! ip forward-protocol nd ! | R2 ! ip vrf cia description CIA Intel rd 1:1 route-target export 1:1 route-target import 1:1 ! ip vrf fbi description FBI Intel rd 1:2 route-target export 1:2 route-target import 1:2 ! interface FastEthernet0/0 bandwidth 10000000 ip address 10.53.123.2 255.255.255.0 ip pim sparse-mode no ip mroute-cache duplex auto speed auto ! interface FastEthernet0/0.11 encapsulation dot1Q 11 ip vrf forwarding cia ip address 192.168.23.2 255.255.255.0 ! interface FastEthernet0/0.12 encapsulation dot1Q 12 ip vrf forwarding fbi ip address 172.30.23.2 255.255.255.0 ! interface FastEthernet0/1 bandwidth 10000000 ip address 10.53.112.2 255.255.255.0 ip pim sparse-mode no ip mroute-cache duplex auto speed auto ! interface FastEthernet0/1.11 encapsulation dot1Q 11 ip vrf forwarding cia ip address 192.168.12.2 255.255.255.0 ! interface FastEthernet0/1.12 encapsulation dot1Q 12 ip vrf forwarding fbi ip address 172.30.12.2 255.255.255.0 ! interface FastEthernet1/0 bandwidth 10000000 ip address 10.53.125.2 255.255.255.0 ip pim sparse-mode no ip mroute-cache duplex auto speed auto ! router eigrp 1 passive-interface FastEthernet1/0 network 0.0.0.0 no auto-summary ! router eigrp 2 auto-summary ! address-family ipv4 vrf cia network 192.168.0.0 0.0.255.255 no auto-summary autonomous-system 2 exit-address-family ! router ospf 1 vrf fbi log-adjacency-changes network 0.0.0.0 255.255.255.255 area 0 ! router rip version 2 network 192.168.0.0 no auto-summary ! address-family ipv4 vrf cia timers basic 5 5 5 5 5 redistribute connected metric 1 network 192.168.0.0 no auto-summary version 2 exit-address-family ! ip forward-protocol nd ! |
| --- | --- |
| R3 ! ip vrf cia description CIA Intel rd 1:1 route-target export 1:1 route-target import 1:1 ! ip vrf fbi description FBI Intel rd 1:2 route-target export 1:2 route-target import 1:2 ! interface FastEthernet0/0 bandwidth 10000000 ip address 10.53.123.3 255.255.255.0 ip pim sparse-mode duplex auto speed auto ! interface FastEthernet0/0.11 encapsulation dot1Q 11 ip vrf forwarding cia ip address 192.168.23.3 255.255.255.0 ! interface FastEthernet0/0.12 encapsulation dot1Q 12 ip vrf forwarding fbi ip address 172.30.23.3 255.255.255.0 ! interface FastEthernet0/1 bandwidth 10000000 ip address 10.53.134.3 255.255.255.0 ip pim sparse-mode duplex auto speed auto ! interface FastEthernet0/1.11 encapsulation dot1Q 11 ip vrf forwarding cia ip address 192.168.34.3 255.255.255.0 ! interface FastEthernet0/1.12 encapsulation dot1Q 12 ip vrf forwarding fbi ip address 172.30.34.3 255.255.255.0 ! interface FastEthernet1/0 bandwidth 10000000 no ip address ip pim sparse-mode shutdown duplex auto speed auto ! router eigrp 1 network 0.0.0.0 no auto-summary ! router eigrp 2 no auto-summary ! address-family ipv4 vrf cia network 192.168.0.0 0.0.255.255 no auto-summary autonomous-system 2 exit-address-family ! router ospf 1 vrf fbi log-adjacency-changes network 0.0.0.0 255.255.255.255 area 0 ! ip forward-protocol nd | R4 ! ip vrf cia description CIA Intel rd 1:1 route-target export 1:1 route-target import 1:1 ! ip vrf fbi description FBI Intel rd 1:2 route-target export 1:2 route-target import 1:2 ! key chain eigrp-key key 1 key-string ccie accept-lifetime 00:00:00 Jan 1 1993 infinite send-lifetime 00:00:00 Jan 1 1993 infinite ! crypto isakmp policy 1 encr aes authentication pre-share group 5 lifetime 60 crypto isakmp key ccie address 10.53.156.6 ! ! crypto ipsec transform-set TS1 esp-aes esp-sha-hmac mode transport ! crypto ipsec profile TUNSEC set security-association lifetime seconds 120 set security-association replay window-size 512 set transform-set TS1 set pfs group5 ! crypto ipsec profile TUNSEC2 set security-association lifetime seconds 120 set security-association replay window-size 512 set transform-set TS1 set pfs group5 ! ! ! ! ip tcp synwait-time 5 ! ! ! ! interface Tunnel0 bandwidth 10000000 ip vrf forwarding cia ip address 192.168.46.4 255.255.255.0 ip authentication mode eigrp 2 md5 ip authentication key-chain eigrp 2 eigrp-key tunnel source 10.53.145.4 tunnel destination 10.53.156.6 tunnel key 0 tunnel protection ipsec profile TUNSEC ! interface Tunnel1 bandwidth 100000 ip vrf forwarding fbi ip address 172.30.46.4 255.255.255.0 tunnel source 10.53.145.4 tunnel destination 10.53.156.6 tunnel key 1 ! interface FastEthernet0/0 bandwidth 10000000 ip address 10.53.145.4 255.255.255.0 ip pim sparse-mode no ip mroute-cache duplex auto speed auto ! interface FastEthernet0/0.11 encapsulation dot1Q 11 ip vrf forwarding cia ip address 192.168.45.4 255.255.255.0 ! interface FastEthernet0/0.12 encapsulation dot1Q 12 ip vrf forwarding fbi ip address 172.30.45.4 255.255.255.0 ! interface FastEthernet0/1 bandwidth 10000000 ip address 10.53.134.4 255.255.255.0 ip pim sparse-mode no ip mroute-cache duplex auto speed auto ! interface FastEthernet0/1.11 encapsulation dot1Q 11 ip vrf forwarding cia ip address 192.168.34.4 255.255.255.0 ! interface FastEthernet0/1.12 encapsulation dot1Q 12 ip vrf forwarding fbi ip address 172.30.34.4 255.255.255.0 ! interface FastEthernet1/0 bandwidth 10000000 no ip address ip pim sparse-mode no ip mroute-cache shutdown duplex auto speed auto ! router eigrp 1 network 0.0.0.0 no auto-summary ! router eigrp 2 auto-summary ! address-family ipv4 vrf cia redistribute connected metric 1 1 1 1 1 network 192.168.0.0 0.0.255.255 no auto-summary autonomous-system 2 exit-address-family ! router ospf 1 vrf fbi log-adjacency-changes network 0.0.0.0 255.255.255.255 area 0 ! ip forward-protocol nd |
| R5 ! interface Tunnel0 bandwidth 10000000 no ip address ! interface FastEthernet0/0 bandwidth 10000000 ip address 10.53.145.5 255.255.255.0 ip pim sparse-mode no ip mroute-cache duplex auto speed auto ! interface FastEthernet0/1 bandwidth 10000000 ip address 10.53.156.5 255.255.255.0 ip pim sparse-mode no ip mroute-cache duplex auto speed auto ! interface FastEthernet1/0 bandwidth 10000000 ip address 10.53.125.5 255.255.255.0 ip pim sparse-mode no ip mroute-cache duplex auto speed auto ! router eigrp 1 passive-interface FastEthernet1/0 network 0.0.0.0 no auto-summary ! ip forward-protocol nd | R6 ! ip vrf cia description CIA Intel rd 1:1 route-target export 1:1 route-target import 1:1 route-target import 1:2 ! ip vrf fbi description FBI Intel rd 1:2 route-target export 1:2 route-target import 1:2 route-target import 1:1 ! key chain eigrp-key key 1 key-string ccie accept-lifetime 00:00:00 Jan 1 1993 infinite send-lifetime 00:00:00 Jan 1 1993 infinite ! crypto isakmp policy 1 encr aes authentication pre-share group 5 lifetime 60 crypto isakmp key ccie address 10.53.145.4 ! ! crypto ipsec transform-set TS1 esp-aes esp-sha-hmac mode transport ! crypto ipsec profile TUNSEC set security-association lifetime seconds 120 set security-association replay window-size 512 set transform-set TS1 set pfs group5 ! crypto ipsec profile TUNSEC2 set security-association lifetime seconds 120 set security-association replay window-size 512 set transform-set TS1 set pfs group5 ! ! ! ! ip tcp synwait-time 5 ! ! ! ! interface Tunnel0 bandwidth 10000000 ip vrf forwarding cia ip address 192.168.46.6 255.255.255.0 ip authentication mode eigrp 2 md5 ip authentication key-chain eigrp 2 eigrp-key tunnel source 10.53.156.6 tunnel destination 10.53.145.4 tunnel key 0 tunnel protection ipsec profile TUNSEC ! interface Tunnel1 bandwidth 100000 ip vrf forwarding fbi ip address 172.30.46.6 255.255.255.0 ip authentication mode eigrp 2 md5 ip authentication key-chain eigrp 2 eigrp-key tunnel source 10.53.156.6 tunnel destination 10.53.145.4 tunnel key 1 ! interface FastEthernet0/0 bandwidth 10000000 no ip address ip pim sparse-mode no ip mroute-cache duplex auto speed auto ! interface FastEthernet0/0.200 encapsulation dot1Q 200 ip address 10.53.200.1 255.255.255.0 ip pim sparse-mode no ip mroute-cache ! interface FastEthernet0/0.201 encapsulation dot1Q 201 ip address 10.53.201.1 255.255.255.0 ip pim sparse-mode no ip mroute-cache ! interface FastEthernet0/1 bandwidth 10000000 ip address 10.53.156.6 255.255.255.0 ip pim sparse-mode no ip mroute-cache duplex auto speed auto ! interface FastEthernet1/0 bandwidth 10000000 ip address 10.53.169.6 255.255.255.0 ip pim bsr-border ip pim sparse-mode duplex auto speed auto ! router eigrp 1 network 0.0.0.0 default-metric 1500 1 255 255 1500 no auto-summary ! router eigrp 2 auto-summary ! address-family ipv4 vrf cia redistribute bgp 1 metric 1 1 1 1 1 network 192.168.0.0 0.0.255.255 no auto-summary autonomous-system 2 exit-address-family ! router ospf 1 vrf fbi log-adjacency-changes redistribute bgp 1 subnets network 0.0.0.0 255.255.255.255 area 0 ! router bgp 1 bgp log-neighbor-changes neighbor 10.53.169.9 remote-as 1 ! address-family ipv4 redistribute eigrp 1 neighbor 10.53.169.9 activate no auto-summary no synchronization exit-address-family ! address-family ipv4 multicast redistribute eigrp 1 route-map multicast-sources neighbor 10.53.169.9 activate no auto-summary no synchronization exit-address-family ! address-family vpnv4 neighbor 10.53.169.9 activate neighbor 10.53.169.9 send-community extended exit-address-family ! address-family ipv4 vrf fbi redistribute ospf 1 vrf fbi neighbor 10.53.169.9 remote-as 1 neighbor 10.53.169.9 activate no synchronization exit-address-family ! address-family ipv4 vrf cia redistribute eigrp 2 neighbor 10.53.169.9 remote-as 1 neighbor 10.53.169.9 activate no synchronization exit-address-family ! ip forward-protocol nd |
| R9 ! ip vrf cia description CIA Intel rd 1:1 route-target export 1:1 route-target export 1:2 route-target import 1:1 route-target import 1:2 ! ip vrf fbi description FBI Intel rd 1:2 route-target export 1:2 route-target export 1:1 route-target import 1:2 route-target import 1:1 ! no ip domain lookup ip multicast-routing ! multilink bundle-name authenticated ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! ! archive log config hidekeys ! ! ! ! ip tcp synwait-time 5 ! ! ! ! interface Loopback0 ip address 10.53.139.39 255.255.255.255 ip pim sparse-mode ! interface Tunnel0 bandwidth 10000000 no ip address ! interface FastEthernet0/0 bandwidth 10000000 ip address 10.53.220.9 255.255.255.0 ip pim sparse-mode duplex auto speed auto ! interface FastEthernet0/1 bandwidth 10000000 no ip address ip pim sparse-mode duplex auto speed auto ! interface FastEthernet1/0 bandwidth 10000000 ip address 10.53.169.9 255.255.255.0 ip pim bsr-border ip pim sparse-mode duplex auto speed auto ! router ospf 1 log-adjacency-changes network 0.0.0.0 255.255.255.255 area 0 ! router bgp 1 bgp log-neighbor-changes neighbor 10.53.169.6 remote-as 1 ! address-family ipv4 redistribute ospf 1 neighbor 10.53.169.6 activate no auto-summary no synchronization exit-address-family ! address-family ipv4 multicast neighbor 10.53.169.6 activate no auto-summary no synchronization exit-address-family ! address-family vpnv4 neighbor 10.53.169.6 activate neighbor 10.53.169.6 send-community extended exit-address-family ! address-family ipv4 vrf fbi neighbor 10.53.169.6 remote-as 1 neighbor 10.53.169.6 activate no synchronization exit-address-family ! address-family ipv4 vrf cia neighbor 10.53.169.6 remote-as 1 neighbor 10.53.169.6 activate no synchronization exit-address-family ! ip forward-protocol nd ! |  |